| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| A | 2026-04-08 | AJ | Initial draft |
| B | 2026-07-23 | AJ | Adjust for UART instead of I2C |

ECP Protocol Specification

This document is the exclusive property of Data Panel Corporation. Any reproduction or this document without permission is prohibited.

Contents

1	Introduction	2

2	Physical layer	3

2.1	Request/Response cycle	3

2.2	I2C bus	4

2.3	Other buses	5

3	Data link layer	5

3.1	Request header	5

3.2	Response Header	6

3.2.1	Result codes	6

3.3	data: Payload	7

3.4	data_crc: Payload CRC	7

4	Network layer	8

4.1	Unconfigured state	8

4.2	Active state	8

4.3	Enumeration and addressing	8

5	Application layer	10

5.1	Host operation	10

5.2	ECP operation	11

6	Commands	12

6.1	Command 0x0000: ECP_CMD_PROTO_VERSION	12

6.2	Command 0x0001: ECP_CMD_IDENT	13

6.3	Command 0x0002: ECP_CMD_ENUM	13

6.4	Command 0x0003: ECP_CMD_HELLO	14

6.5	Command 0x0004: ECP_CMD_ECHO	14

6.6	Command 0x0005: ECP_CMD_REBOOT	14

6.7	Command 0x0006: ECP_CMD_FEATURES	15

6.8	Command 0x0007: ECP_CMD_DEVICE_EVENT	15

6.9	Command 0x0008: ECP_CMD_ESTOP	16

6.10	Command 0x0009: ECP_CMD_BOOT_JUMP	16

6.11	Command 0x1000: ECP_CMD_IO_READ	17

6.12	Command 0x1001: ECP_CMD_IO_WRITE	18

6.13	Command 0x1002: ECP_CMD_IO_GET_ATTRIB	18

6.14	Command 0x1003: ECP_CMD_IO_SET_ATTRIB	19

6.15	Command 0x1100: ECP_CMD_LED_COLOR	19

6.16	Command 0x1101: ECP_CMD_LED_BRIGHTNESS	20

6.17	Command 0x1200: ECP_CMD_GPIO_CONFIG	20

6.18	Command 0x1201: ECP_CMD_GPIO	20

6.19	Command 0xFF00: ECP_CMD_PEEK	21

6.20	Command 0xFF01: ECP_CMD_POKE	22

6.21	Command 0xFF02: ECP_CMD_COMM_STATS	22

6.22	Command 0xFF03: ECP_CMD_UPTIME	22

7	Features	23

7.1	ECP_FEATURE_CH_IO	23

7.2	ECP_FEATURE_PEEK_POKE	23

7.3	ECP_FEATURE_GPIO	23

7.4	ECP_FEATURE_DEVICE_EVENT	24

7.5	ECP_FEATURE_COMM_STATS	24

7.6	ECP_FEATURE_LED	24

8	Reference	25

8.1	DPPort attributes	25

# Introduction

The Embedded Co-Processor (ECP) protocol is a proprietary protocol designed to enable intra-board communication over a serial connection between a primary MCU (the host) and one or more I/O co-processors (ECP). The protocol aims to achieve the following goals:

- Accommodate new features without breaking backward compatibility

- Be as simple as possible without needlessly sacrificing flexibility

- Work well over I2C, but allow for other physical layers

- Support automatic identification and enumeration of ECPs and their features

- Allow at least 16 ECPs to be connected to a single host

The following assumptions are made:

- ECPs are connected to no more than one host at a time.

- The host MCU can individually reset each ECP

- The host MCU has configuration-related information that the ECP does needs but does not have access to

This document does not describe the bootloader protocol used to load firmware onto the ECP. For STM32-based ECPs over I2C, see ST Application Note AN4221: I2C protocol used in the STM32 bootloader.

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”,“SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY” and “OPTIONAL” in this document are to be interpreted as described in RFC-2119.

# Physical layer

The primary physical layer for the ECP protocol is a UART. ECPs MUST support communication at speeds of at least 1 Mbps.

An alternate I2C physical layer was evaluated, but may cause latency problems with many ECPs. If using I2C, all devices MUST support Fast Mode+ at up to 1 Mbit/sec.

## Request/Response cycle

Each request/response cycle consists of a request message containing a command and a response from the corresponding ECP. Both the request and response include optional data. The request is an I2C write transaction, and the response is an I2C read transaction, regardless of the direction of the flow of data. A request with no response is an incomplete transaction.

The host MAY interleave transactions to different ECPs such that multiple transactions are pending at a time.

Figure 1:Interleaved transactions

Hosts MUST keep track of all pending requests they submit to ECPs until the response is successfully received by the host. ECPs MUST keep track of transactions directed at their configured address, but can ignore all other I2C traffic.

## UART

During the request the host transmits data to the ECP and the ECP receives data.

For the response, the roles are reversed and the ECP transmits data to the host.

## The ECP SHOULD NOT transmit data to the host except as a response to a request.

## I2C bus

Figure 2: Simple ECP network

The host acts as I2C bus master, and the ECP acts as the slave device. Each request/response cycle consists of two I2C transactions: an I2C write from the host to the ECP (the request), and an I2C read (host reading from ECP).

As the bus master, the host initiates all transactions including requests and responses. Because ECPs must not initiate transactions they have no means of contacting the host asynchronously. Thus, ECPs SHOULD provide an out-of-band mechanism to indicate a need for service to the host, such as an interrupt GPIO. The interrupt line MAY be shared among several ECPs if required. The host can then query each ECP to determine required actions.

The two parts of the transaction SHOULD separated by a STOP condition or alternatively, a repeated START condition MAY be used.

Figure 3: Request I2C transaction, Host → ECP

Figure 4: Response I2C transaction, ECP → Host

ECPs MUST NOT block transactions while a request is being processed, for example by pulling SCL low (“clock stretching”). Instead, ECPs MUST process requests in the background. Long-running commands may need additional time to complete. The ECP SHOULD provide a temporary response indicating that the command is being processed, but is not yet complete. Once complete, the ECP SHALL prepare a response message to be read by the host.

If the host attempts to read data beyond the end of the prepared response message, the ECP SHALL return a NULL byte (0x00) for each byte read.

The ECP SHALL discard any bytes received beyond the length indicated by the header. If the request header is invalid, the ECP SHALL ignore all subsequent bytes received until transaction is complete.

## Other buses

The protocol design may be adapted to other physical layers such as SPI. This document does not address such use cases.

# Data link layer

Figure 5: Header overview

Request and response frames consist of a header along with an optional CRC-protected data payload.

The header is always six bytes in length but has a slightly different structure in request and response frames.

Both headers include a CRC8 using the following parameters:

| Polynomial | 0x97 |
| --- | --- |
| Initial value | 0x00 |
| XOR Out | 0x00 |

This CRC is also known as CRC-8-AUTOSAR.

## 3Request header

Figure 6: Request header structure

| Field | Description |
| --- | --- |
| seq_dup | sequence duplicate flag; indicates re-transmission of previous request |
| seq_number | Sequence number; used by the host to correlate responses with requests |
| is_response | 0 for requests |
| version | Protocol structure version |
| command | command identifier |
| data_len | length of data associated with request, up to 1023 bytes |
| command_version | revision level of command |
| header_crc | CRC8 of header data |

## Response Header

Figure 7: Response header structure

| Field | Description |
| --- | --- |
| seq_dup | sequence duplicate flag; indicates re-transmission of previous response |
| seq_number | Sequence number; used by the host to correlate responses with requests |
| is_response | 1 for responses |
| version | Protocol structure version |
| result | Result code |
| data_len | length of data associated with response, up to 1023 bytes |
| header_crc | CRC8 of header data |

### Result codes

| Code | Name |  |  | Description |
| --- | --- | --- | --- | --- |
| 0x00 | ECP_RES_SUCCESS |  |  | Request completed successfully |
| 0x01 | ECP_RES_INVALID_COMMAND |  |  | Command not supported |
| 0x02 | ECP_RES_ERROR |  |  | Request failed for other unspecified reason |
| 0x03 | ECP_RES_INVALID_PARAM |  |  | Request contained invalid or out of range parameter |
| 0x04 | ECP_RES_NOT_ALLOWED |  |  | Command not allowed in current state |
| 0x05 | ECP_RES_UNSUPPORTED_CMD_VERSION |  |  | Unsupported command version |
| 0x06 | ECP_RES_NOT_SUPPORTED |  | Command not implemented for provided parameters |  |
| 0x07 | ECP_RES_BUSY |  |  | A conflicting command is still pending |
| 0x08 | ECP_RES_TIMEOUT |  |  | Timeout while processing |
| 0x09 | ECP_RES_OVERFLOW |  |  | Data overflow |
| 0x0A | ECP_RES_INVALID_HEADER |  |  | Header has invalid or out of range data |
| 0x0B | ECP_RES_REQUEST_TRUNCATED |  |  | Request was incomplete |
| 0x0C | ECP_RES_RESPONSE_TOO_BIG |  |  | Response generates too much data |
| 0x0D | ECP_RES_BUS_ERROR |  |  | General communication bus error |
| 0x0E | ECP_RES_INVALID_HEADER_VERSION |  |  | Unsupported protocol version |
| 0x0F | ECP_RES_INVALID_HEADER_CRC |  |  | Header CRC did not match |
| 0x10 | ECP_RES_INVALID_DATA_CRC |  |  | Data CRC did not match |
| 0x11 | ECP_RES_DUP_UNAVAILABLE |  |  | Unable to re-send response |
| 0x12 | ECP_RES_STOPPED |  |  | Emergency stop. Reset required |
| 0x13 … 0xFD | Reserved |  |  |  |
| 0xFE | ECP_RES_IN_PROGRESS |  |  | Command accepted and is being processed |
| 0xFF | ECP_RES_UNAVAILABLE |  |  | No result available |

## data: Payload

The payload contains one or more bytes as arguments or outputs of the command. The data field MUST NOT be included in the frame if data_length is zero.

All devices MUST support data payloads up to the maximum payload size.

## data_crc: Payload CRC

This field is a 16-bit CRC over the payload data. The following parameters are used:

| Polynomial | 0x1021 |
| --- | --- |
| Initial value | 0xFFFF |
| XOR Out | 0x0000 |

This CRC is known as CRC16/CCITT-FALSE and is defined in ITU-T Recommendation V.41.

The data_crc field MUST only be included in the frame if data_length is non-zero. A frame with a data_length of zero and an included data_crc is not valid. ECPs SHOULD response to such messages with a result code of ECP_RES_OVERFLOW.

Devices MUST validate any received data against the data CRC and MUST reject any frames with invalid data CRCs.

# Network layer

The ECP is always in one of two states: unconfigured or active. The ECP automatically enters the unconfigured state on power up. Upon successful initialization, the ECP moves to the active state. Command availability may vary by state.

## Unconfigured state

In the unconfigured state, If using I2C, the ECP uses a fixed 7-bit I2C address of 0x40. The ECP enters the unconfigured state immediately after powering up.

Some commands such as the I/O commands are not available in the unconfigured state because they require information that may not be available before enumeration.

## Active state

The active state is the normal operational state of the ECP. If using I2C, in this state the ECP uses a 7-bit I2C address of 0x50 offset by a value specified by the host. The offset corresponds to the port index such that the first ECP uses 0x50, the second uses 0x51, and so on.

The ECP enters the active state after receiving a valid ECP_CMD_ENUM message. The process by which the host configures the ECPs is called enumeration.

## Enumeration and addressing

🗒️NOTE

On a shared communication bus such as I2C, the host MUST have individual control of the reset signal of each ECP so that the host can address each ECP in sequence. Depending on the hardware design, additional I/O may be required such as bootloader control pins.

Because each ECP in a device may have identical firmware and identical circuits, a method is needed to configure each ECP at runtime. The host enables each ECP in turn and using a known, default bus address configures the ECP and re-assigns it a unique address. The host then repeats the process for each ECP in turn. The host MAY automatically load or update the firmware of each ECP as part of the enumeration process.

A flow chart depicting the enumeration process is shown below.

Figure 8: Enumeration of individual ECP

# Application layer

## Host operation

Figure 9: Host algorithm for transmitting requests

## ECP operation

Figure 10: ECP algorithm for processing requests

Figure 11: ECP request processing state machine

# Commands

All command identifiers are 16-bit values. Commands taking arguments or returning response data specify the structure of the payload data. If not specified, the command has no associated data.

## Command 0x0000: ECP_CMD_PROTO_VERSION

This command gets the protocol version. This revision of the protocol always returns 2.

| Command ID | 0x0000 |
| --- | --- |
| Name | Protocol version |
| Command revision | 2 |
| Request payload format | None |
| Response payload format | struct { uint8_t versions; uint16_t max_request_packet_size; uint16_t max_response_packet_size; uint16_t flags; }; |
| Required features | None |
| Availability | ✅Unconfigured ✅Active |

🗒️NOTE

“In-progress” support allows the host to query the status of long-running background tasks I2C-based ECPs SHOULD implement IN_PROGRESS_SUPPORTED.

| Flag bit | Description |  |
| --- | --- | --- |
| 0 | IN_PROGRESS_SUPPORTED |  |

## Command 0x0001: ECP_CMD_IDENT

This command gets the software version information for the ECP.

| Command ID | 0x0001 |
| --- | --- |
| Name | Software Identification |
| Command revision | 1 |
| Request payload format | struct { uint8_t major; uint8_t minor; uint16_t patch; uint32_t build; uint8_t sw_part_number[16]; }; |
| Response payload format | struct { uint8_t major; uint8_t minor; uint16_t patch; uint32_t build; uint8_t sw_part_number[16]; }; |
| Required features | None |
| Availability | ✅Unconfigured ✅Active |

## Command 0x0002: ECP_CMD_ENUM

This command instructs the ECP to set its address and enter active mode. The ECP uses the type and revision fields to aid hardware configuration.

| Command ID | 0x0002 |
| --- | --- |
| Name | Enumerate |
| Command revision | 2 |
| Request payload format | struct { uint8_t offset; uint8_t ecp_type; uint8_t ecp_revision; }; |
| Response payload format | None |
| Required features | None |
| Availability | ✅Unconfigured ❌Active |

| # | ECP Type |
| --- | --- |
| 1 | ECP_TYPE_DO_DI_5A |
| 2 | ECP_TYPE_DO_DI_13A |
| 3 | ECP_TYPE_DOPLUS_DI |
| 4 | ECP_TYPE_ADI |

## Command 0x0003: ECP_CMD_HELLO

This command verifies that the ECP is responsive to commands. The ECP SHALL return in_data + 0x01020304.

| Command ID | 0x0003 |
| --- | --- |
| Name | Hello |
| Command revision | 1 |
| Request payload format | struct { uint32_t in_data; }; |
| Response payload format | struct { uint32_t out_data; }; |
| Required features | None |
| Availability | ✅Unconfigured ✅Active |

## Command 0x0004: ECP_CMD_ECHO

This command is like ECP_CMD_HELLO, but supports arbitrary data lengths. Unlike ECP_CMD_HELLO, the ECP returns unmodified data from the request. This is useful for stress testing the ECP communication bus at varying packet lengths.

| Command ID | 0x0004 |
| --- | --- |
| Name | Echo |
| Command revision | 1 |
| Request payload format | struct { uint8_t data[]; }; |
| Response payload format | struct { uint8_t data[]; }; |
| Required features | None |
| Availability | ✅Unconfigured ✅Active |

## Command 0x0005: ECP_CMD_REBOOT

This command restarts the ECP immediately after the response is read. The ECP will need to be re-enumerated, and all I/O channels will need to be re-configured.

| Command ID | 0x0005 |
| --- | --- |
| Name | Reboot |
| Command revision | 1 |
| Request payload format | None |
| Response payload format | None |
| Required features | None |
| Availability | ✅Unconfigured ❌Active |

## Command 0x0006: ECP_CMD_FEATURES

This command returns the supported features. Feature support is indicated by setting the corresponding bit. See Features for details.

| Command ID | 0x0006 |
| --- | --- |
| Name | Get supported features |
| Command revision | 1 |
| Request payload format | None |
| Response payload format | struct { uint32_t flags; }; |
| Required features | None |
| Availability | ✅Unconfigured ✅Active |

## Command 0x0007: ECP_CMD_DEVICE_EVENT

Query and configure device events. Device events can be used by the host to quickly check for what needs to be serviced after the ECP asserts the interrupt GPIO.

| Value | Operation |
| --- | --- |
| 0 | ECP_DEVICE_EVENT_OP_GET_CURRENT_EVENTS |
| 1 | ECP_DEVICE_EVENT_OP_GET_ENABLED_EVENTS |
| 2 | ECP_DEVICE_EVENT_OP_SET_ENABLED_EVENTS |

| Bit | Event | Description | Default |
| --- | --- | --- | --- |
| 0 | ECP_DEVICE_EVENT_BOOT | Set on boot. Can be used to detect software restarts. |  |
| 1 | ECP_DEVICE_EVENT_THERMAL | Set when temperature threshold exceeded | X |
| 2 | ECP_DEVICE_EVENT_IO_FAULT | Set when one or more channels detects a fault | X |

The status of all events is reflected in the value of event_mask, but only enabled events trigger the interrupt GPIO output.

| Command ID | 0x0007 |
| --- | --- |
| Name | Device Events |
| Command revision | 1 |
| Request payload format | struct { uint32_t event_mask; uint8_t operation; }; |
| Response payload format | struct { uint32_t event_mask; }; |
| Required features | ECP_FEATURE_DEVICE_EVENT |
| Availability | ✅Unconfigured ✅Active |

## Command 0x0008: ECP_CMD_ESTOP

❗ IMPORTANT

If multiple ECPs are sent the ESTOP command on a shared bus such as I2C, the only reliable way to recover is to re-enumerate all ECPs.

This command immediately returns all I/O to a safe state and enters the unconfigured state. This command has no effect when the ECP is already in the unconfigured state.

| Command ID | 0x0008 |
| --- | --- |
| Name | Emergency stop |
| Command revision | 1 |
| Request payload format | None |
| Response payload format | None |
| Required features | None |
| Availability | ✅Unconfigured ✅Active |

## Command 0x0009: ECP_CMD_BOOT_JUMP

This command instructs the ECP to enter the bootloader immediately after the response is read. The ECP will no longer be available until it is restarted.

| Command ID | 0x0009 |
| --- | --- |
| Name | Jump to bootloader |
| Command revision | 1 |
| Request payload format | None |
| Response payload format | None |
| Required features | None |
| Availability | ✅Unconfigured ❌Active |

## Command 0x1000: ECP_CMD_IO_READ

Get the value of an I/O channel.

| Command ID | 0x1000 |
| --- | --- |
| Name | Get channel state |
| Command revision | 2 |
| Request payload format | struct { uint8_t ch; } |
| Response payload format | struct { uint8_t ch; uint8_t mode; uint32_t raw; int32_t scaled; int32_t voltage; // feedback mV, or -1 int32_t current; // feedback mA, or -1 int32_t temperature; // 0.001 °C, // or -1 if not equipped uint8_t fault_status; uin8t_t hb_direction; uin8t_t flags; } |
| Required features | ECP_FEATURE_CH_IO |
| Availability | ❌Unconfigured ✅Active |

The raw and scaled data depends on the active mode:

| Mode | Raw | Scaled |
| --- | --- | --- |
| Disable | -1 | -1 |
| PWMOUT | Scaled, ramped duty cycle (MIN – MAX) | Commanded duty cycle (0-10000) |
| PWMIOUT | Commanded duty cycle | Setpoint (mA) |
| DIN_POS | 0/1 | Same as raw |
| FREQIN_POS | Input freq. (mHz) | Measured duty cycle (0-10000) |
| COUNT_POS | Counts (+ only) | Same as raw |
| ENCODER_POS | Counts (+/-) | Same as raw |
| DIN_GND | 0/1 | Same as raw |
| FREQIN_GND | Input freq. (mHz) | Measured duty cycle (0-10000) |
| COUNT_GND | Counts (+ only) | Same as raw |
| ENCODER_GND | Counts (+/-) | Same as raw |
| AIN_0_5_V | ADC counts | Voltage (mV) |
| AIN_0_10_V | ADC counts | Voltage (mV) |
| AIN_0_32_V | ADC counts | Voltage (mV) |
| AIN_4_20_MA | ADC counts | Current (μA) |
| R_IN | ADC counts | Resistance (mΩ) |

The flags field is encoded as follows:

| Bit | Name |  | Description |
| --- | --- | --- | --- |
| 7:6 | Channel Status |  | 00: Inactive 01: Active 02: Error/Fault 03: Not available |
| 5:4 | Sensor Power Status |  | 00: Inactive 01: Active 02: Error/Fault 03: Not available |
| 3:2 | Soft switch status |  | 00: Inactive 01: Active 02: Error/Fault 03: Not available |
| 1 | Paused |  | 0: Not paused 1: Paused |
| 0 | Reserved |  | Always 0 |

## Command 0x1001: ECP_CMD_IO_WRITE

Set the value of an output channel. Interpretation of value depends on the channel type and configured mode.

| Command ID | 0x1001 |
| --- | --- |
| Name | Write channel state |
| Command revision | 1 |
| Request payload format | struct { uint8_t ch; int32_t value; } |
| Response payload format | None |
| Required features | ECP_FEATURE_CH_IO |
| Availability | ❌Unconfigured ✅Active |

| Mode | Value | Units |
| --- | --- | --- |
| Disable | Ignored |  |
| PWMOUT | Commanded duty cycle | 0 – 10000 |
| PWMIOUT | Setpoint | mA |
| DIN_POS | Sensor power enable |  |
| FREQIN_POS | Sensor power enable |  |
| COUNT_POS | Sensor power enable |  |
| ENCODER_POS | Sensor power enable |  |
| DIN_GND | Sensor power enable |  |
| FREQIN_GND | Sensor power enable |  |
| COUNT_GND | Sensor power enable |  |
| ENCODER_GND | Sensor power enable |  |
| AIN_0_5_V | Sensor power enable |  |
| AIN_0_10_V | Sensor power enable |  |
| AIN_0_32_V | Sensor power enable |  |
| AIN_4_20_MA | Sensor power enable |  |
| R_IN | Sensor power enable |  |

## Command 0x1002: ECP_CMD_IO_GET_ATTRIB

Get an attribute of an I/O channel. Attribute identifiers are defined by the DPUnity port API. Interpretation of the value is attribute specific.

| Command ID | 0x1002 |
| --- | --- |
| Name | Get channel attribute |
| Command revision | 1 |
| Request payload format | struct { uint8_t ch; uint8_t attrib_id; }; |
| Response payload format | struct { uint8_t ch; uint8_t attrib_id; uint32_t value; }; |
| Required features | ECP_FEATURE_CH_IO |
| Availability | ❌Unconfigured ✅Active |

## Command 0x1003: ECP_CMD_IO_SET_ATTRIB

Set an attribute of an I/O channel Attribute identifiers are defined by the DPUnity port API. Interpretation of the value is attribute specific.

| Command ID | 0x1003 |
| --- | --- |
| Name | Set channel attribute |
| Command revision | 1 |
| Request payload format | struct { uint8_t ch; uint8_t attrib_id; uint32_t value; }; |
| Response payload format | None |
| Required features | ECP_FEATURE_CH_IO |
| Availability | ❌Unconfigured ✅Active |

## Command 0x1100: ECP_CMD_LED_COLOR

Set the color(s) of an LED.

| Command ID | 0x1100 |
| --- | --- |
| Name | Set LED color |
| Command revision | 1 |
| Request payload format | struct { uint8_t led; // LED index uint8_t color_count; // 3 for RGB uint8_t colors[]; // color intensity, 0-255 }; |
| Response payload format | None |
| Required features | ECP_FEATURE_LED |
| Availability | ❌Unconfigured ✅Active |

## Command 0x1101: ECP_CMD_LED_BRIGHTNESS

Set the brightness of an LED.

| Command ID | 0x1101 |
| --- | --- |
| Name | Set LED brightness |
| Command revision | 1 |
| Request payload format | struct { uint8_t led; // LED index (0xFF for all) uint8_t brightness; // overall brightness, 0-255 }; |
| Response payload format | None |
| Required features | ECP_FEATURE_LED |
| Availability | ❌Unconfigured ✅Active |

## Command 0x1200: ECP_CMD_GPIO_CONFIG

Configure available GPIO. The mask field indicates which bits of the direction field should be changed. Use a mask of 0x0000 to retrieve the current configuration without modifying it. Use a mask of 0xFFFF to configure all GPIO.

Default GPIO configuration depends on ECP implementation. GPIO mapping and availability depends on ECP implementation.

The ECP SHALL ignore attempts to configure unsupported GPIO.

| Command ID | 0x1200 |
| --- | --- |
| Name | Configure GPIO |
| Command revision | 1 |
| Request payload format | struct { uint16_t direction; // 0: output, 1: input uint16_t mask; }; |
| Response payload format | struct { uint16_t direction; // 0: output, 1: input }; |
| Required features | ECP_FEATURE_GPIO |
| Availability | ✅Unconfigured ✅Active |

## Command 0x1201: ECP_CMD_GPIO

Get/set available GPIO. The mask field indicates which bits of the outputs field should be changed. Use a mask of 0x0000 to retrieve the current values without modifying them. Use a mask of 0xFFFF to write all outputs.

Default GPIO configuration depends on ECP implementation. GPIO mapping and availability depends on ECP implementation.

The ECP SHALL ignore attempts to set an unsupported GPIO. The ECP SHALL ignore attempts to set GPIO configured as inputs.

| Command ID | 0x1201 |
| --- | --- |
| Name | Get/Set GPIO |
| Command revision | 1 |
| Request payload format | struct { uint16_t outputs; uint16_t mask; }; |
| Response payload format | struct { uint16_t inputs; }; |
| Required features | ECP_FEATURE_GPIO |
| Availability | ✅Unconfigured ✅Active |

## Command 0xFF00: ECP_CMD_PEEK

❗ IMPORTANT

This command has security implications.

This command allows the host to read arbitrary data from the RAM of the ECP for debugging purposes. The ECP MAY restrict access to certain memory regions. The ECP MAY require specific byte alignment. Certain error conditions might cause the ECP to reset.

| Command ID | 0xFF00 |
| --- | --- |
| Name | Peek, memory read |
| Command revision | 1 |
| Request payload format | struct { uint32_t address; uint16_t count; }; |
| Response payload format | struct { uint8_t data[]; }; |
| Required features | ECP_FEATURE_PEEK_POKE |
| Availability | ✅Unconfigured ✅Active |

## Command 0xFF01: ECP_CMD_POKE

❗ IMPORTANT

This command has security implications.

This command allows the host to write arbitrary data into the RAM of the ECP for debugging purposes. The ECP MAY restrict access to certain memory regions. The ECP MAY require specific byte alignment. Certain error conditions might cause the ECP to reset.

| Command ID | 0xFF01 |
| --- | --- |
| Name | Poke, memory write |
| Command revision | 1 |
| Request payload format | struct { uint32_t address; uint16_t count; uint8_t data[]; }; |
| Response payload format | None |
| Required features | ECP_FEATURE_PEEK_POKE |
| Availability | ✅Unconfigured ✅Active |

## Command 0xFF02: ECP_CMD_COMM_STATS

This command returns statistics related to the ECP communication interface.

| Command ID | 0xFF02 |
| --- | --- |
| Name | Communication statistics |
| Command revision | 1 |
| Request payload format | None |
| Response payload format | struct { uint32_t request_count; uint32_t error_count; uint32_t dup_count; uint32_t request_bytes; uint32_t response_bytes; }; |
| Required features | ECP_FEATURE_COMM_STATS |
| Availability | ✅Unconfigured ✅Active |

## Command 0xFF03: ECP_CMD_UPTIME

This command returns the uptime of the ECP and the reset cause. Implementations SHOULD identify the cause of the last reset. Implementations MUST use a reset reason of 0 (UNKNOWN) if the cause cannot be determined.

| Value | Reset reason |
| --- | --- |
| 0 | UNKNOWN |
| 1 | POWER |
| 2 | BROWNOUT |
| 3 | TRAP |
| 4 | EXTERNAL |
| 5 | SOFTWARE |
| 6 | WATCHDOG |
| 7 | OTHER |

| Command ID | 0xFF03 |
| --- | --- |
| Name | Uptime |
| Command revision | 1 |
| Request payload format | None |
| Response payload format | struct { uint32_t uptime_ms; uint8_t reset_reason; }; |
| Required features | None |
| Availability | ✅Unconfigured ✅Active |

# Features

This document describes all currently defined messages, but individual ECP implementations may not implement all features. Reasons for not supporting a feature include memory constraints or features intended for non-production testing and development.

| Bit | Name | Description |
| --- | --- | --- |
| 0 | ECP_FEATURE_CH_IO | DPUnity port channel I/O |
| 1 | ECP_FEATURE_PEEK_POKE | Direct memory access |
| 2 | ECP_FEATURE_GPIO | Read/write individual I/O pins |
| 3 | ECP_FEATURE_DEVICE_EVENT | Events from ECP to host |
| 4 | ECP_FEATURE_COMM_STATS | Keep track of ECP communication errors |
| 5 | ECP_FEATURE_LED | RGB LED control |
| 6..31 | Reserved |  |

## ECP_FEATURE_CH_IO

This feature supports the use of the DPUnity port I/O API. Nearly all ECPs will implement this feature.

## ECP_FEATURE_PEEK_POKE

This feature allows direct access to memory on the ECP and thus is not intended to be enabled in production builds. It is useful for development and troubleshooting in a lab environment.

## ECP_FEATURE_GPIO

This feature allows control of specific pins of the ECP that may not be associated with the port I/O. This feature is mostly intended for debugging, but could also be used by the host to read configuration pins or perform other functions.

## ECP_FEATURE_DEVICE_EVENT

This feature allows the host to monitor one or more events that happen on the ECP. This is used by the host to identify what needs servicing after a shared interrupt is sent by an ECP.

## ECP_FEATURE_COMM_STATS

This feature allows tracking of communication errors.

## ECP_FEATURE_LED

This feature supports control of user-visible LED indicators.

# Reference

## DPPort attributes

NOTE: The C enumeration for each value below is prefixed with PORT_CH_ATTRIB_.

| # | Name | Description |
| --- | --- | --- |
| 0 | MODE | I/O channel mode |
| 1 | PID_TIME | PID loop period, in ms |
| 2 | PID_KP | PID proportional gain constant |
| 3 | PID_KI | PID integral gain constant |
| 4 | PID_KD | PID derivative gain constant |
| 5 | PWM_MIN_DUTY | Minimum PWM output duty cycle |
| 6 | PWM_MAX_DUTY | Maximum PWM output duty cycle |
| 7 | PWM_RAMP_TIME_UP | Time to ramp from minimum duty to maximum duty |
| 8 | PWM_RAMP_TIME_DOWN | Time to ramp from maximum duty to minimum duty |
| 9 | OUTPUT_CURRENT_SOFT_LIMIT | Threshold for i2t letthrough |
| 10 | OUTPUT_CURRENT_TRIP_TIME | Trip time for i2c letthrough monitoring |
| 11 | OUTPUT_OVERCURRENT_REACTION | Determines behavior when letthrough limit is reached |
| 12 | OUTPUT_CURRENT_LIMIT_MAX_RETRIES | Number of times to allow retrying |
| 13 | COUNTER_EN | Deprecated |
| 14 | COUNTER_ENABLE_OUT | Deprecated |
| 15 | COUNTER_OVERFLOW_ENABLE | If set, the counter will “overflow” back to zero when set point is reached. |
| 16 | COUNTER_SET_POINT | Maximum value for counter before saturation or overflow |
| 17 | HBRIDGE_INVERT_DIRECTION | Change sense of current flow in h-bridge outputs |
| 18 | SENSOR_POWER_BOOT_ENABLE | Not used on ECP |
| 19 | SENSOR_POWER_VOLTAGE_SELECT | Selects sensor power output (5 V or Vbat) |
| 20 | DIGIN_ANALOG_THRESHOLD_LOW | Threshold for configurable digital inputs, including frequency |
| 21 | DIGIN_ANALOG_THRESHOLD_HIGH | Threshold for configurable digital inputs, including frequency |
| 22 | ANALOG_INPUT_FILTER_CONSTANT | Filter constant for analog inputs and output current feedback |
| 23 | DISABLE_CURRENT_CHOPPING | Disables current chopping on supported outputs |

## Result code equivalent POSIX errno

## The table below maps ECP result codes into system error numbers based on The Open Group Base Specifications Issue 8, 2024 edition (IEEE 1003.1-2024) – also known as POSIX.1-2024. The system error numbers are found in the errno.h C header file.

| Result code | POSIX Error Number (errno) |  |
| --- | --- | --- |
| ECP_RES_SUCCESS | 0 |  |
| ECP_RES_INVALID_COMMAND | -EPROTO |  |
| ECP_RES_ERROR | -EIO |  |
| ECP_RES_INVALID_PARAM | -EINVAL |  |
| ECP_RES_NOT_ALLOWED | -EPERM |  |
| ECP_RES_UNSUPPORTED_CMD_VERSION |  | -EPROTO |  |
| ECP_RES_NOT_SUPPORTED | -ENOTSUP |  |
| ECP_RES_BUSY | -EBUSY |  |
| ECP_RES_TIMEOUT | -ETIMEDOUT |  |
| ECP_RES_OVERFLOW | -EOVERFLOW |  |
| ECP_RES_INVALID_HEADER | -EIO |  |
| ECP_RES_REQUEST_TRUNCATED | -EPROTO |  |
| ECP_RES_RESPONSE_TOO_BIG | -E2BIG |  |
| ECP_RES_BUS_ERROR | -EFAULT |  |
| ECP_RES_INVALID_HEADER_CRC | -EIO |  |
| ECP_RES_INVALID_DATA_CRC | -EIO |  |
| ECP_RES_DUP_UNAVAILABLE | -ENOMSG |  |
| ECP_RES_STOPPED | -EHOSTDOWN |  |
| ECP_RES_IN_PROGRESS | -EINPROGRESS |  |
| ECP_RES_UNAVAILABLE | -ENOENT |  |

# Revision History