# Shell command reference (ECP)

## Debug console

| Device | lpuart1, pins PB11 (TX) and PB10 (RX) |
| --- | --- |
| Speed | 115,200 bps |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |

The firmware provides a serial console for debugging. Several commands are included to facilitate design validation and debugging. These commands are described in the sections below.

Access the serial console using a tool like PuTTY or TeraTerm.

NOTE: Due to memory constraints, the ECP console is significantly less featureful than the main micro’s console.

### Starting the ECP

Depending on what software is on the main MCU, you may need to enable the ECP by executing the following commands on the main MCU’s console. The values below are for ECP 1. Adjust the I/O pin numbers based on which ECP is under test.

uart:~$ gpio conf tcal9539@74 0 ol1

Put the ECP in reset (drive low)

uart:~$ gpio conf tcal9539@74 9 i

Release BOOT0

uart:~$ gpio conf tcal9539@74 0 i

Release the ECP reset (high-Z)

NOTE: If the ECP reset signal or BOOT0 is being actively driven, STM32CubeProgrammer will not be able to program the ECP.

You may also want to enable VLOGIC_VSENSE by enabling the MCU_KEEPALIVE signal from the main MCU.

uart:~$ board keepalive on

## Basic commands

This section describes some commands included with Zephyr that may be useful.

### Read/write arbitrary memory

Device register memory can be read/written using the devmem command.

uart:~$ devmem 0x40022020 32
Using data width 32
Read value 0xfefffeaa

For example, to read the nBOOT_SEL field from the OPTR register (address 0x40022020):

uart:~$ devmem 0x40022020 32 0xfefffeaa
Using data width 32
Writing value 0xfefffeaa

Similarly, to write memory:

Reading or writing arbitrary memory can have undesired effects including crashes.  In such cases, the watchdog will restart the system after the timeout.

## Kernel

### Stack usage

uart:~$ kernel stacks
0x20000618 NA (real size 2048): unused 1628     usage 420 / 2048 (20 %)
0x20000498 NA (real size 2048): unused 1176     usage 872 / 2048 (42 %)
0x200008e8 NA (real size 1024): unused 696      usage 328 / 1024 (32 %)
0x200006d0 NA (real size 256):  unused 168      usage 88 / 256 (34 %)
0x20000778 NA (real size 1024): unused 704      usage 320 / 1024 (31 %)
0x200031e0 IRQ 0     (real size 2048):  unused 1792     usage 256 / 2048 (12 %)

Thread stack usage can be displayed using the kernel stacks command.  The entry point of each thread is displayed along with the allocated and used stack in bytes as well as percent.

### Uptime

uart:~$ kernel uptime
Uptime: 535758 ms

The time since last power-up can be displayed using kernel uptime:

### Thread information

Detailed information on each active thread, including CPU usage and stack usage, can be displayed with the kernel threads command:

uart:~$ kernel threads
Scheduler: 11 since last call
Threads:
 0x20000618 NA
        options: 0x1, priority: 4 timeout: 13
        state: suspended, entry: 0x800377d
        stack size 2048, unused 1628, usage 420 / 2048 (20 %)

*0x20000498 NA
        options: 0x0, priority: 14 timeout: 0
        state: queued, entry: 0x8002799
        stack size 2048, unused 1176, usage 872 / 2048 (42 %)

0x200008e8 NA
        options: 0x1, priority: -1 timeout: 0
        state: pending, entry: 0x8007eed
        stack size 1024, unused 696, usage 328 / 1024 (32 %)

0x200006d0 NA
        options: 0x1, priority: 15 timeout: 0
        state: , entry: 0x800aeef
        stack size 256, unused 168, usage 88 / 256 (34 %)

0x20000778 NA
        options: 0x1, priority: 0 timeout: 377
        state: suspended, entry: 0x8007aed
        stack size 1024, unused 704, usage 320 / 1024 (31 %)

## Sensors

An overview of sensor values can be shown using the sensors command.

uart:~$ sensors
VLOAD_VSENSE=0.0 V
TEMP=25.163 °C

CSENSE_OUT_A=0.0 A   (0.494 V)
FAULT_OUT_A=0 (raw=1)
VSENSE_OUT_A=0 (raw=1)
ANALOG_IN_A=0.0 V

CSENSE_OUT_B=0.0 A   (0.497 V)
FAULT_OUT_B=0 (raw=1)
VSENSE_OUT_B=0 (raw=1)
ANALOG_IN_B=0.0 V

## Outputs

### Output status

uart:~$ port status
PORT_1 type=1 channels=2
    PORT_1A fault_status=OK (0)
            status=0
            value=0
            raw_value=0
            current=0 mA
            temperature=25 C
    Attributes
            [0] MODE=12 (0xc)
            [1] PWM_FREQ=250 (0xfa)
            [6] PWM_MIN_DUTY=0 (0x0)
            [7] PWM_MAX_DUTY=10000 (0x2710)
            [8] RAMP_TIME_FWD=2000 (0x7d0)
            [9] RAMP_TIME_REV=2000 (0x7d0)
            [10] OUTPUT_CURRENT_SOFT_LIMIT=13000 (0x32c8)
            [11] OUTPUT_CURRENT_TRIP_TIME=250 (0xfa)
            [12] OUTPUT_OVERCURRENT_REACTION=0 (0x0)
            [13] OUTPUT_CURRENT_LIMIT_MAX_RETRIES=5 (0x5)
            [23] ANALOG_INPUT_FILTER_CONSTANT=8192 (0x2000)
    PORT_1B fault_status=OK (0)
            status=0
            value=0
            raw_value=0
            current=0 mA
            temperature=25 C
    Attributes
            [0] MODE=12 (0xc)
            [1] PWM_FREQ=250 (0xfa)
            [6] PWM_MIN_DUTY=0 (0x0)
            [7] PWM_MAX_DUTY=10000 (0x2710)
            [8] RAMP_TIME_FWD=2000 (0x7d0)
            [9] RAMP_TIME_REV=2000 (0x7d0)
            [10] OUTPUT_CURRENT_SOFT_LIMIT=13000 (0x32c8)
            [11] OUTPUT_CURRENT_TRIP_TIME=250 (0xfa)
            [12] OUTPUT_OVERCURRENT_REACTION=0 (0x0)
            [13] OUTPUT_CURRENT_LIMIT_MAX_RETRIES=5 (0x5)
            [23] ANALOG_INPUT_FILTER_CONSTANT=8192 (0x2000)

Query the status of the output channels with the port status command.

### Set output

uart:~$ port write 1 1 5000

Change the output PWM duty cycle using the port write <port> <channel> <value> command, where port is 1, channel is 1 (channel A) or 2 (channel B), and value is the duty cycle 0-10,000.

### Configure port

uart:~$ port seta 1 1 2 347

Change the output configuration using the port seta <port> <channel> <attrib> <value> command, where port is 1, channel is 1 (A) or 2 (B), attrib is the attribute identifier (see table) and value is the new value of the attribute.  For example, to change channel A PWM frequency to 347 Hz:

| ### Attr. # | ### Attribute | ### Default | ### Minimum | ### Maximum |
| --- | --- | --- | --- | --- |
| ### 0 | ### MODE | ### 12 (PWM) |  |  |
| ### 1 | ### PWM_FREQ | ### 250 Hz | ### 25 Hz | ### 2500 Hz |
| ### 6 | ### PWM_MIN_DUTY | ### 0 | ### 0 | ### 10000 |
| ### 7 | ### PWM_MAX_DUTY | ### 10000 | ### 0 | ### 10000 |
| ### 8 | ### RAMP_TIME_UP | ### 2000 ms | ### 0 ms | ### 10000 ms |
| ### 9 | ### RAMP_TIME_DOWN | ### 2000 ms | ### 0 ms | ### 10000 ms |
| ### 10 | ### OUTPUT_CURRENT_SOFT_LIMIT | ### 13000 mA | ### 0 mA | ### 13000 mA |
| ### 11 | ### OUTPUT_CURRENT_TRIP_TIME | ### 250 ms | ### 1 ms | ### 5000 ms |
| ### 12 | ### OUTPUT_OVERCURRENT_CURRENT_REACTION | ### 0 (fault) |  |  |
| ### 13 | ### OUTPUT_OVERCURRENT_MAX_RETRIES | ### 5 | ### 0 | ### 250 |
| ### 23 | ### ANALOG_INPUT_FILTER_CONSTANT | ### 8192 | ### 0 | ### 65535 |

### Clear output fault

To clear an output channel’s fault, use the port clear <port> <channel> command.

uart:~$ port clear 1 1

NOTE: This command will have no effect if the output fault cannot be cleared, for example when the fault is still present or the maximum number of retries has been met.